SECTION 081116 - ALUMINUM DOORS AND FRAMES (Architectural Terrace)

*TO FINISH YOUR SPECIFICATION: 1. Please read, react to, then delete comments in red or *stars*. 2. Please add or delete options in green or (parentheses). 3. Please replace preceding standards with options in blue [brackets] or delete options in blue [brackets]. 4. Finally, please revise paragraph numbers and/or letters, as necessary.*

*BEFORE USING YOUR FINISHED SPECIFICATION: Permit Peerless to review your edits to confirm your selected options do not restrict your desired operation or reduce the tested performance.*

# PART 1 - GENERAL

1.01  SECTION INCLUDES

- Material: aluminum doors as on the drawings and specified in this section.

- Installation: labor, tools, and material needed to install aluminum doors.

- Glass and glazing.

(1.02  PRODUCTS SUPPLIED BUT NOT INSTALLED)

*Enter description, e.g., extra door panels to be supplied and stored for the future*

(1.03  PRODUCTS INSTALLED BUT NOT SUPPLIED)

*Enter description, e.g., louver supplied by others to be installed in new door panel*

1.04  RELATED SECTIONS - Section 079200 – Joint Sealants

1.05  REFERENCES

- AAMA - American Architectural Manufacturers Association - www.aamanet.org

- AAMA/WDMA/CSA 101/I.S.2/A440-08 “NAFS - North American Fenestration Standard/Specification for windows, doors, and skylights”

- AAMA 502-12 "Voluntary Specification for Field Testing of Newly Installed Fenestration Products"

- AAMA 611-12 "Voluntary Specification for Anodized Architectural Aluminum"

- AAMA 701/702-11 "Voluntary Specification for Pile Weatherstripping and Replaceable Fenestration Weatherseals”

- AAMA 910-10 “Voluntary ‘Life Cycle’ Specifications and Test Methods for AW Class Architectural Windows and Doors”

- AAMA 920-11 “Specification for Operating Cycle Performance of Side-Hinged Exterior Door Systems”

- AAMA 1304-02 “Voluntary Specification for Forced Entry Resistance of Side-Hinged Door Systems”

- AAMA 2605-13 “Voluntary Specification, Performance Requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels”

- AAMA CW-10-12 "Care and Handling of Architectural Aluminum from Shop to Site"

- ASTM - American Society for Testing and Materials – www.astm.org

- ASTM E283-12 "Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen"

- ASTM E330-14 "Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights, and Curtain Walls by Uniform Static Air Pressure Difference"

- ASTM E331-09 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Uniform Static Air Pressure Difference"

- ASTM E547-09 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential"

- ASTM E2190-10 "Standard Specification for Insulating Glass Unit Performance and Evaluation"

- IGCC – Insulating Glass Certification Council – www.igcc.org

- NFRC – National Fenestration Rating Council – www.nfrc.org

  - NFRC 100-2010 “Procedure for Determining Fenestration Product U Factors”

  - NFRC 102-2010 “Procedure for Measuring the Steady-State Thermal Transmittance of Fenestration Systems”

- SGCC – Safety Glazing Certification Council – www.sgcc.org

  - ANSI Z97.1-09 “American National Standard for Safety Glazing Materials used in Buildings – Safety Performance Specifications and Methods of Test”

  - 16 CFR 1201 “Consumer Product Safety Commission Safety Standard for Architectural Glazing Materials – codified at Title 16, Part 1201 of the Code of Federal Regulations 2011 Edition”

1.06  SYSTEM DESCRIPTION

- AAMA Designation: AW-PG100-ATD.

- Doors: 3-1/4" frame depth; extruded aluminum with integral structural thermal break installed by the door manufacturer in the frame and panel members; equal-leg frame; exterior and interior finishes applied by the door manufacturer; frames and panels assembled by the door manufacturer.

- Thermal break: The thermal break separating the exterior and interior aluminum extrusions shall be a mechanical crimp-in-place system utilizing multi-directional glass fiber reinforced polyamide nylon struts with locking mechanical connections to the aluminum extrusions. The thermal break shall not be compromised by hardware or metal fasteners.

- Configuration: inswing door panel; [panel-to-panel in one master frame;] [panel-to-fixed in one master frame;] [panel-under-fixed in one master frame;] panel prehung in door frame; panel hinged at the jamb and swings in to open.

- Handing: left hand – GTD2-61 = the hinges are on left, when standing outside and looking in (; right hand – GTD2-65 = the hinges are on the right, when standing outside and looking in).

- Glazing: exterior ExxonMobil Santoprene™ foam gasket; 1" insulating glass made with mandatory *Enter either tempered or laminated* safety glass lites; two weep holes under each glass pocket for drainage; foam backer rod and silicone heel bead forming an internal seal; interior Santoprene™ bulb gasket threaded into aluminum glazing beads; glass description in paragraph 2.04; glazed by the door manufacturer.

1.07 PERFORMANCE REQUIREMENTS

- Conformance to AW-PG100-ATD specifications in AAMA/WDMA/CSA 101/I.S.2/A440-08 when tests are performed on the prescribed 48” x 96" minimum test size with the following test results:

- Air Infiltration: after the AAMA 910-10 life cycle and AAMA 920-03 operating cycle tests, not to exceed the standard of maximum .1 cfm/square foot when tested per ASTM E283-12 at a static air pressure difference of 6.24 psf.

- Water Penetration with standard non-ADA sill: after the AAMA 910-10 life cycle and AAMA 920-03 operating cycle tests, no uncontrolled water leakage when tested per ASTM E331-09 and ASTM E547-09 at a static air pressure difference of 12 psf.

- Uniform Deflection: no more than L/175 when tested per ASTM E330-14 at a static air pressure difference of 100 psf.

- Uniform Structural Load: no glass breakage or permanent damage to fasteners, and maximum .2% permanent deformation of the span of any frame member when tested per ASTM E330-14 at a static air pressure difference of 150 psf.

- Forced Entry Resistance: latching devices shall provide reasonable security against forced entry and the test door shall achieve ‘No Entry’ when tested per AAMA 1304-02.

- Thermal NFRC Simulation: thermal computer simulation per NFRC 100-2010 on a 48” x 96" test size glazed with 1” insulating made with 1/4” soft coat low E coating on surface #2, argon gas in the airspace made with a stainless steel spacer, and 1/4” clear glass, with the following test result: Standardized Thermal Transmittance to be maximum 0.351 BTU/HR/SQ.FT/°F.

1.08  SUBMITTALS

- Shop drawings: door location chart; typical door elevations; details of assemblies, hardware, and glazing details for units glazed by the door manufacturer.

- Product data: test reports from an AAMA-accredited laboratory; manufacturer's specifications.

- Samples: each specified finish for aluminum; other samples as requested.

1.09  QUALITY ASSURANCE

- Submit for prebid approval ten days prior to bid opening valid test reports or waiver from an AAMA-accredited laboratory conforming to test results in Paragraph 1.07, and a sample door representing the bid door except for color.

- Acceptance will be by addendum only as no verbal approvals will be allowed.

- Submit bid on prequalified products in prebid written addendum. Bidder must identify manufacturer and model of product on which the bid is based.

- Furnish a valid AAMA “Authorization for Product Certification” or waiver indicating that the doors for the project conform to AAMA/WDMA/CSA 101/I.S.2/A440-08.

- Furnish visible, permanent IGCC certification labels indicating compliance with ASTM E 2190-10 on the insulating glass units.

- Furnish visible, permanent SGCC certification labels indicating conformance to ANSI Z97.1-09 and/or 16 CFR 1201 on tempered glass lites, if included on the project, and laminated glass lites, if included on the project.

- Manufacturer's warranties:

- Doors: warrant for one year against defects in material or workmanship under normal use.

- Insulating glass units: warrant seal for ten years against visual obstruction from film formation or moisture collection between internal glass surfaces, excluding that caused by glass breakage or abuse.

- Paint finish: Finish conforming to AAMA 2605-13: warrant for twenty years against chipping, peeling, blistering, cracking, chalking, or fading.

(H. Project Survey: *Contact Peerless to register before project bid date* by installer

and manufacturer’s representatives; one year after date of completion; to

recommend maintenance procedures.)

  - DELIVERY, STORAGE, AND HANDLING - Handle and protect doors and

accessories in accordance with AAMA CW-10-12 until installation completion.

# PART 2 - PRODUCTS

2.01  MANUFACTURERS

- Peerless GTD2-61 Left Hand (GTD2-65 Right Hand) Thermal Aluminum Inswing Terrace Door

- Other acceptable manufacturers who have demonstrated a successful history of manufacturing for *Enter number* years equivalent products:

- *Enter an equivalent manufacturer’s company name and model number*

- *Enter another equivalent manufacturer’s company name and model number*

2.02  MATERIALS

- Aluminum extrusions: extruded by the door manufacturer from commercial quality 6063-T5 alloy; free from defects impairing strength and durability.

- Hardware: mounted in concealed extruded grooves to avoid penetrating frame or panel members with fasteners; three exposed aluminum knuckle hinges per panel to rotate the panel inward on vertical axis; one single-action satin nickle [black] handle with multiple point locks per panel; [one key-operated black (silver) (white) handle with multiple point locks per panel;] [one custodial-operated satin nickle (black) (white) handle with multiple point locks per panel and security wrench tool;] (key-release limit arms *specify clear opening dimension*;).

- Weatherstrip: secured in extruded ports; Santoprene™ bulb seal conforming to AAMA 702-11: flap seal single row mounted on the vent interior on the bottom rail, top rail, and stiles; triple-tubular seal single internal row mounted on the head, sill and jamb frame members.

2.03 FABRICATION

- Frame: double tubular head, sill, and jambs mitre cut and fastened with two zamac corner gussets per corner; double tubular integral mullion, if required, fastened with two zamac gussets per frame member without penetrating the frame member with fasteners; corners sealed by the door manufacturer with polyethylene foam gaskets.

- Panel: double tubular horizontal and vertical vent rails and stiles mitre cut and fastened with two zamac corner gussets per corner; corners sealed by the door manufacturer with polyethylene foam gaskets.

- Water control: continuous compression gasket within the internal frame to utilize pressure equalization and to allow water to drain by gravity through exterior covered weep slots covered with weep flaps and weep hoods finished to match window.

2.04  INSULATING GLASS UNITS

### Materials

- Spacer: tubular stainless steel.

- Spacer color: stainless metal color.

- Primary seal: polyisobutylene.

- Secondary seal: silicone.

- Airspace fill: plain air. [argon.]

### Performance

1.	Dual-seal durability: conformance to ASTM E 2190-10; visible, permanent IGCC certification label.

(2. Other: *Enter center-of-glass U value, SHGC, and VT data as required*)

- Exterior glass lite

- Thickness: 1/8". [3/16".] [1/4".]

- Tint: clear. [bronze.] [gray.] [green.]

- Type: tempered. [laminated. *Enter interlayer and lite descriptions*]

(4.  Coating: low E coating on #2 surface; Cardinal Glass 270 or Cardinal Glass

366 or equivalent.)

- Interior glass lite

- Thickness: 1/8". [3/16".] [1/4".]

- Tint: clear. [obscure. *Enter pattern*]

- Type: tempered. [laminated. *Enter interlayer and lite descriptions*]

(4.  Coating: soft low E coating on #3 surface; Cardinal Glass 270 or Cardinal

Glass 366 or equivalent.)

(5.  Coating: low E coating on #4 surface; Cardinal Glass i89 or equivalent.)

- Glass surface protection: Factory-apply removeable protective clear Cardinal Glass Preserve™ film on insulating glass exterior and interior surfaces for protection during window installation and building construction.

2.05  FINISH ON EXTERIOR ALUMINUM EXTRUSIONS

- Application: on clean extrusions free from serious surface blemishes; on exposed surfaces visible when installed product's operating panel is closed.

*Enter information for the exterior finish from one of the three finishes listed below*

*Enter the following for an AAMA 2605 70% fluoropolymer paint finish*

- Coating: Resin containing 70% fluoropolymer; thermosetting; alternative finishes will not be acceptable.

- Quality standard: conforming to AAMA 2605-13, including 10 years Florida exposure and 4000 hours humidity tests.

- Pretreatment: five-stage; zinc chromate conversion coating.

- Application: electrostatic spray by an Approved Applicator and appropriate oven bake process.

- Coating quantity: minimum one primer coat and one color coat.

- Dry film thickness: minimum 1.2 mils on exposed surfaces, except inside corners and channels.

- Color: chosen from manufacturer's standards.

*Or enter the following for an AAMA 611 clear anodize finish*

- Coating: clear anodize.

- Quality standard: conforming to AAMA 611-12.

- Thickness: AAM10C22A41 Class I - .7 mils #215. [AAM10C22A31 Class II - .4 mils #204.]

*Or enter the following for an AAMA 611 color anodize finish*

- Coating: color anodize.

- Quality standard: conforming to AAMA 611-12.

- Thickness: AAM10C22A44 Class I - .7 mils.

- Color: #313 dark bronze. [#311 light bronze.] [#312 medium bronze.] [#315 black.]

2.06  FINISH ON INTERIOR ALUMINUM EXTRUSIONS

- Application: on clean extrusions free from serious surface blemishes; on exposed surfaces visible when installed product's operating panel is closed.

*Enter information for the interior finish from one of the three finishes listed below*

*Enter the following for an AAMA 2605 70% fluoropolymer paint finish*

- Coating: Resin containing 70% fluoropolymer; thermosetting; alternative finishes will not be acceptable.

- Quality standard: conforming to AAMA 2605-13, including 10 years Florida exposure and 4000 hours humidity tests.

- Pretreatment: five-stage; zinc chromate conversion coating.

- Application: electrostatic spray by an Approved Applicator and appropriate oven bake process.

- Coating quantity: minimum one primer coat and one color coat.

- Dry film thickness: minimum 1.2 mils on exposed surfaces, except inside corners and channels.

- Color: chosen from manufacturer's standards.

*Or enter the following for an AAMA 611 clear anodize finish*

- Coating: clear anodize.

- Quality standard: conforming to AAMA 611-12.

- Thickness: AAM10C22A41 Class I - .7 mils #215. [AAM10C22A31 Class II - .4 mils #204.]

*Or enter the following for an AAMA 611 color anodize finish*

- Coating: color anodize.

- Quality standard: conforming to AAMA 611-12.

- Thickness: AAM10C22A44 Class I - .7 mils.

- Color: #313 dark bronze. [#311 light bronze.] [#312 medium bronze.] [#315 black.]

(2.07  MUNTINS)

- Material: extruded aluminum; with exposed surfaces finished to match panel color; concealed fasteners; designed for unrestricted expansion and contraction.

- Design: muntin bar cross-section profile and material chosen from manufacturer’s standards.

- Patterns: grid patterns to be designated by architect.

- Location: exterior. (internal: encapsulated between the two glass lites in the insulating glass unit to protect them from damage and dirt buildup.) [true: dividing the insulating glass into small individual units.]

(2.08  INSTALLATION ACCESSORIES)

- Material: extruded aluminum; nominal .062” wall; with exposed surfaces finished to match door color and finish performance; concealed fasteners; required weatherseals; designed for unrestricted expansion and contraction.

- Exterior: (snap-in sill panning leg.) (wrap around panning.) (preset panning.) (snap-in nailing fin.) (snap-in flange.) (two-piece receptor with polyamide strip thermal break.) (10 psf subsill with polyamide strip thermal break and end dams sealed by the door manufacturer.)

- Interior: (two-piece snap trim and trim clip.)

- Mullions: (horizontal stack mullion with polyamide strip thermal break.) (vertical three-piece mullion with polyamide strip thermal break.)

- Other: (steel strap anchor.) (head expander with polyamide strip thermal break.) (sill angle.)

PART 3 - EXECUTION

3.01  PREPARATION - Prepare openings to be in tolerance, plumb, level,  provide for

secure anchoring, and in accordance with approved shop drawings.

3.02  INSTALLATION

- Install doors in accordance with approved shop drawings and door manufacturer's recommendations with skilled craftspeople who have demonstrated a successful history of installing doors for *Enter number* years.

- Provide required support and securely fasten and set doors plumb, square, and level without twist or bow.

- Apply sealant per sealant manufacturer's recommendations at joints, wipe off excess, and leave exposed sealant surfaces clean and smooth.

(3.03  FIELD TESTING)

- Test installed units in conformance with AAMA 502-12 minimum requirements for air and water infiltration with the door manufacturer, dealer, contractor, and owner present.

- Select test units as directed by the owner's representative and use an AAMA-accredited laboratory provided by the owner or contractor.

3.04  ADJUSTING  - Adjust doors as necessary for smooth and weathertight

operation.

3.05  CLEANING

- Peel the Preserve™ protective film from the door glass surfaces when the door installation is complete, leaving these surfaces ready for use without the need for glass cleaning.

- Leave the installed doors clean and free of construction debris.

3.06  CARE PRIOR TO BUILDING COMPLETION - Protect surfaces of installed

doors, in accordance with AAMA CW-10-12, from contact with

contaminating substances resulting from surrounding construction.

END OF SECTION